⚡ Qfficient: System Manual & Technical Reference

Thermodynamic-Mapped QUBO for Dynamic Microgrid Distribution
Next-Generation Power Routing via Non-Equilibrium Thermodynamics & Quantum Annealing

1. Executive Summary & Problem Context

Modern electrical distribution microgrids are experiencing rapid deployment of intermittent Distributed Energy Resources (DERs)—such as rooftop solar PV arrays, wind turbines, community battery storage (BESS), and ultra-fast electric vehicle (EV) charging depots.

• The Classical Breakdown: Traditional distribution network reconfiguration (DNR) relies on static shortest-path algorithms or Mixed-Integer Non-Linear Programming (MINLP). These classical methods assume fixed conductor resistances (R0) and take seconds to minutes to converge—failing to respond during steep, transient renewable surges (di/dt and dP/dt).
• Thermal Runaway: Real conductors heat up quadratically (P = 3 I² R). High line temperatures trigger a positive thermal feedback loop where conductor resistance rises non-linearly, escalating thermal degradation and permanently destroying available electrical work (exergy).
• The Qfficient Innovation: We reframe dynamic network reconfiguration strictly as an entropy-generation minimization problem grounded in non-equilibrium thermodynamics, compiling dynamic conductor temperatures and physical line flows directly into the Ising spin glass Hamiltonian of a quantum annealer.

PHYSICS PRINCIPLE: Gouy-Stodola Theorem: The rate of usable electrical exergy destroyed is strictly proportional to the thermodynamic entropy generated: Ex_destroyed = T0 · ∫ ∑ S_gen(t) dt. Minimizing entropy generation directly maximizes delivered grid capacity.

2. Comprehensive Technology Stack Breakdown

| Layer / Subsystem | Technologies & Libraries | Version | Role in Qfficient |
| --- | --- | --- | --- |
| Language & Runtime | Python (CPython) | 3.10 / 3.11 | Core execution environment, asynchronous task handling, and typing. |
| Scientific Computing | NumPy, SciPy | >=1.26, >=1.11 | Vectorized branch current solver, matrix QUBO evaluation, paired t-tests. |
| Graph & Network Theory | NetworkX | >=3.0 | Feeder graph topology, fundamental cycle detection, and spanning tree checks. |
| Data Handling | Pandas | >=2.0.0 | Telemetry logging, injection profile management, and tabular exports. |
| Quantum Formulation | dimod | >=0.12.0 | Binary Quadratic Models (BQM), QUBO-to-Ising mapping (x = (s+1)/2). |
| Quantum Annealing | OpenJij | >=0.12.0 | Simulated Quantum Annealing (SQA) engine with transverse field sweeps. |
| Classical Annealing | dwave-neal | >=0.6.0 | Classical simulated annealing benchmark sampler. |
| Hardware QPU Access | dwave-samplers / Ocean | >=1.8.0 | Leap API token integration for physical D-Wave Advantage QPUs. |
| Power Flow Engine | LinDistFlow (FastFeeder) | Internal | Linearized AC distribution power flow calculating currents, losses, and voltages. |
| Thermal Physics | Differential Heat Balance | Internal | C_th dT/dt = 3 I² R - ΔT/θ_th with non-linear temperature coefficient α. |
| Interactive GUI | Streamlit | >=1.35.0 | Full presentation dashboard with interactive sliders and real-time report cards. |
| Data Visualization | Matplotlib | >=3.8.0 | 300-DPI publication plots, 4-panel transient dynamics, topology schematics. |
| CLI Formatting | tabulate | >=0.10.0 | Formatted ASCII/Unicode terminal tables for simulation summaries. |
| Testing & Verification | pytest | >=8.0.0 | 17 unit & physics verification tests enforcing radiality and math integrity. |
| Containerization | Docker & Compose | Production | Multi-stage slim container based on python:3.11-slim with health monitoring. |

3. Core Mathematical & Thermodynamic Formulations

1. Entropy Generation Rate:
   S_gen,e(t) = Q_dot_e(t) / T_e(t) = [3 · I_e(t)² · R_e(T_e(t))] / T_e(t)  [W/K]

2. Exergy Destruction (Gouy-Stodola):
   Ex,destroyed = T0 · ∫ ∑ S_gen,e(t) dt   (where T0 = 298.15 K)

3. Conductor Transient Heat Balance Equation:
   C_th,e · (dT_e/dt) = 3 · I_e(t)² · R_e(T_e(t)) - [T_e(t) - T_amb] / θ_th,e
   R_e(T_e) = R0,e · [1 + α · (T_e - T0)], where α ≈ 0.00393 K⁻¹ (copper/aluminum)

4. Master Thermodynamic QUBO Hamiltonian submitted to Annealer:
   H(x) = λ_thermo · H_thermo(x) + λ_cycle · H_cycle(x) + λ_rad · H_rad(x) + λ_cap · H_cap(x) + λ_sw · H_sw(x)

   - H_thermo = ∑ w_e x_e, where w_e = (3 I_e² R_e) / T_e
   - H_cycle = ∑_k (∑_{e ∈ C_k} x_e - (|C_k| - 1))²  (Cycle-breaking penalty)
   - H_rad = (∑ x_e - (|V| - N_sub))²  (Spanning tree radiality)
   - H_cap = Heavily penalizes branches exceeding thermal ampacity I_max
   - H_sw = Penalizes excessive breaker toggling between consecutive steps

4. Repository Structure & Codebase Blueprint

qfficient/ ├── app.py # Interactive Streamlit Web Portal & Report Card UI ├── main_experiment.py # Primary simulation & 4-panel transient dynamics figure generator ├── run_benchmark.py # Multi-scenario statistical benchmark & ablation suite ├── run_custom_grid.py # Scalable custom microgrid runner (10 to 100 components) ├── run_custom_40.py # Preset runner for 40-component microgrid ├── run_custom_60.py # Preset runner for 60-component microgrid ├── visualize_microgrid.py # 12-component microgrid topology diagram generator ├── run_dashboard.bat # 1-Click launcher for Windows ├── run_dashboard.sh # 1-Click launcher for Linux/macOS ├── pyproject.toml # Package metadata & console entrypoints ├── requirements.txt # Pinned dependency requirements ├── Dockerfile # Production container specification (Python 3.11-slim) ├── docker-compose.yml # Multi-container service orchestrator ├── README.md # Executive project documentation & research outcomes ├── MANUAL.md # Markdown operations manual ├── results/ # Benchmark reports, telemetry JSON, and distribution plots ├── tests/ # Pytest test suite (17 passed tests) └── thermodynamic_grid_qubo/ # Core library package ├── pipeline.py # Unified optimization pipeline & config dataclasses ├── grid_engine/ # Power grid topologies and thermal dynamics ├── qubo_engine/ # QUBO and Ising Hamiltonian compilation ├── solvers/ # Optimization backends (SQA, Neal, NumPy, Oracle) └── verification/ # LinDistFlow solver, metrics, and visualizer

5. Complete CLI Command Reference & User Operations Manual

5.1 Interactive Web Dashboard (Streamlit)

Launch the interactive review portal with live parameter sliders and report cards:

# Direct Streamlit execution: streamlit run app.py # Or using the virtual environment directly: .venv\Scripts\python.exe -m streamlit run app.py # 1-Click launchers: run_dashboard.bat # Windows ./run_dashboard.sh # Linux/macOS

5.2 Main Scientific Experiment (IEEE 33-Bus Transient)

Executes the synchronized transient benchmark (75% solar drop + 800 kW EV charging pulse) and generates 'thermodynamic_qubo_results.png':

# Direct python execution: python main_experiment.py # Or via registered console entrypoint: thermo-qubo-sim

5.3 Scalable Custom Microgrid Runner (10 to 100 Buses)

Simulates arbitrary custom microgrid topologies generated via graph algorithms:

# Basic usage (default 60 components, 6 ties, 15 steps): python run_custom_grid.py # 40 components with 4 tie lines: python run_custom_grid.py --components 40 --ties 4 # 80 components with 8 tie switches over 20 time steps: python run_custom_grid.py --components 80 --ties 8 --steps 20 # 100 components using Exact Oracle solver: python run_custom_grid.py --components 100 --ties 8 --solver exact_oracle # Using the vectorized NumPy annealer: python run_custom_grid.py --components 50 --ties 5 --solver numpy_annealer # Ready-made benchmark presets: python run_custom_40.py python run_custom_60.py

5.4 Multi-Scenario Benchmark & Policy Ablation Suite

Runs multi-scenario statistical ablation studies with Student's t-test verification:

# Run benchmark with 5 scenarios: python run_benchmark.py --n 5 # Comprehensive benchmark with 20 scenarios saved to custom directory: python run_benchmark.py --n 20 --output-dir results

5.5 Microgrid Topology Visualizer

Generates high-resolution schematic diagrams highlighting active paths vs. open switches:

python visualize_microgrid.py # Or via console entrypoint: thermo-qubo-gui

5.6 Automated Test Suite Execution (pytest)

Executes 17 comprehensive unit, physical verification, and schema validation tests:

# Run all tests: .venv\Scripts\python.exe -m pytest # Verbose output: .venv\Scripts\python.exe -m pytest -v # Run specific test modules: .venv\Scripts\python.exe -m pytest tests/test_custom_grid.py .venv\Scripts\python.exe -m pytest tests/test_fast_flow.py .venv\Scripts\python.exe -m pytest tests/test_pipeline.py .venv\Scripts\python.exe -m pytest tests/test_qubo_grid.py

6. Solver Matrix & Performance Comparison

| Solver Identifier | Algorithm Description | Loss Reduction | Voltage Compliant | Latency |
| --- | --- | --- | --- | --- |
| exact_dynamic_oracle | Global loss-minimizing dynamic branch-exchange | +28.37% | Yes (100%) | 38.98 ms |
| static_loss_optimal | Optimal static spanning tree configuration | +27.87% | Yes (100%) | 0.00 ms |
| sqa (OpenJij) | Simulated Quantum Annealing (Transverse Ising) | Dynamic | Yes (100%) | ~330 ms |
| numpy_annealer | Vectorized local-search simulated annealing | Dynamic | Yes (100%) | ~15 ms |
| dwave_qpu_runner | Physical D-Wave Advantage QPU (Leap Cloud) | Dynamic | Yes (100%) | Cloud |
| standard_static | Unreconfigured textbook baseline (tie lines open) | 0.00% | Baseline | 0.00 ms |

7. Programmatic Python API & Extensibility

You can integrate Qfficient directly into external scripts or Jupyter notebooks:

from thermodynamic_grid_qubo.pipeline import OptimizationConfig, run_optimization # 1. Configure the scenario config = OptimizationConfig( grid='custom', solver='exact_oracle', solar_drop_pct=75.0, ev_surge_kw=800.0, ambient_c=25.0, time_steps=20, custom_n_components=50, custom_n_ties=5, ) # 2. Run optimization results = run_optimization(config) # 3. Access physics telemetry b_heat = sum(results['baseline']['heat_mj']) q_heat = sum(results['qubo']['heat_mj']) pct_saved = ((b_heat - q_heat) / b_heat) * 100.0 print(f'Joule Heat Reduction: {pct_saved:+.2f}%') print(f'Voltage Compliant: {results["checks"]["voltage_all_steps"]}')

8. Embedded Visual Diagrams & Schematics

Figure 1: 12-Component Smart Microgrid Dynamic Reconfiguration Routing Diagram:

Figure 1. Active energized feeder paths (solid green) vs. standing open candidate tie lines (dashed gray).

Figure 2: Transient Dynamics & Loss Reduction on IEEE 33-Bus Benchmark:

Figure 2. 4-Panel publication transient analysis: (A) Active power loss, (B) Cumulative Joule heat dissipation, (C) Entropy generation rate, and (D) Peak feeder conductor temperature.